You can change the time before proceeding to rotate the bucket. The screenshot above is located on line 19. To change the time or delay:

8000 miliseconds is  equal to 8 seconds. Always give a mili value denoting a factor of one thouasanth (10^-3) . Example:

5 seconds = 5000

10 seconds = 10000

50 seconds = 50000

Please don’t add (,) or (.) like:

5 seconds = 5,000

10 seconds = 10,000

50 seconds = 50.000

In line 63, you can adjust the value according to your preferred distance inside the bucket. I've set the distance to less than 33 cm using "if (distance < 33)", allowing it to only detect distances lower than 33 cm. Avoid using the greater than symbol. The Sonar has a distance limit between 2 to 400 cm. I conducted several trials concerning the sensing distance inside the bucket, especially when a cat passes through it.

Initially, I set it to 45 cm "if (distance < 45)", but it detected directly to the lid of the bucket (yung takip), which triggered the green LED, so I recommend trial and error.

In this line, this is the milisecond of delay. I set it in 1 second before changing its rotation from clockwise to counterclockwise position.

Other instructions:

- 1. Please use a higher voltage and current below 9 volts.

- 2. Please reset the arduino if sudden issues or errors occur. Arduino-Uno is not always perfect, especially when using clone boards, so expect some issues.

- If errors persist, please follow these steps:

- - Check if the batteries are still at 80 to 90 percent capacity/charge. Huwag gamitin kapag lobat na ang mga batterya.

- - Thoroughly check the wiring, pins, and components.

- - Review any modifications made in the program code.

- - Avoid using Chinese or clone Arduinos. While they can be reliable for budget projects, for better output, please use an original Arduino board.

- - Use small servo motor that has an input of 4.8 to 6 v.

- - Use serial USB port for additional voltage need of the prototype.